Creating and managing ecrS WI 1046343 REV G - Accuray Confidential Copyright Ó 2025 Accuray Incorporated

Revision History

| Rev | Change # | Description of Change | Revised By | Date |
| --- | --- | --- | --- | --- |
| E | DCO-00585 | Create section for External Change requests. Replace Supply Chain Management with Global Sourcing throughout. | Sarah Schmidt | 07-Jul-2022 |
| F | CHG-00222 | Clarification of ECR process scope. Revision Change Review Board Process. Change verbiage from CCB to CRB. | Nathan Shirey | 01-May-2024 |
| G | CHG-01171 | Consolidate “Sustaining CCB” and “Other CCBs” | Nathan Shirey | See DMS |

Table of Contents

1	Purpose	4

2	Scope	4

3	Reference Documents	4

4	Definitions	4

5	Procedure	5

5.1	Scope of ECR Process	5

5.2	Change Review Board Process	6

5.3	Process	8

5.4	ECR Creation Instructions – All CRBs	12

5.5	Agile Workflow – All CRBs	19

5.6	Other Procedures using the ECR Process	21

6	Records	22

7	Appendix	22

# Purpose

The purpose of this procedure is to provide instructions for preparation, processing, and submittal of Engineering Change Requests (ECRs).

# Scope

This procedure applies to change requests submitted and reviewed through the Agile ECR process for all ECR Change Review Board (CRB) options. This procedure applies to Accuray employees who have been granted the 02 ECO Creator, 02 ECR Creator, or 02 ECR Coordinator roles in Agile. This procedure is within the scope of SOP 016827. This procedure does not apply to DCRs, see WI 1066466.

# Reference Documents

- SOP 016827 Change Control (Parent SOP)

- SOP 016487 Supplier Quality

- SOP 016824 Quality Records and Records Retention

- SOP 1044943 Post Market Surveillance

- SOP 1060697 Supplier Change Request (SCR)

- WI 032394 Creating ECO Items and Changes

- WI 1044497 Processing Service Change Requisition Submissions

- WI 1063933 Product Change Review Board

- WI 1066466 Creating DCRs

- FRM 033017 Initial Change Assessment

# Definitions

See Accuray Acronyms and Definitions (1089757).

# Procedure

## Scope of ECR Process

The Engineering Change Request WI applies to all parts and documents that are directly related to design, purchasing, assembly, and shipping to customers. This work instruction applies to all items in Agile that have the subclass shown in the table in Section 5.4.2.6. Items of other subclasses (e.g., QMS docs) follow WI 1066466. For additional information, also refer to WI 032394.

The ECR process is intended to identify opportunities, problems, or issues that may not have a known solution, so ECR creators are not required to provide perfect information before submitting an ECR. Provide the details that are available, and as needed, the ECR can be assigned for investigation to acquire additional information. The ECR is the plan for what will happen.

Entering ECRs for work not already contained within approved projects allows functions to triage, track, and complete requests. ECRs are submitted for many reasons, including the following:

### Any issue has been identified in a product or part.

### Any service issue or concern has been identified with a product, part, FRU, or service documentation.

### Any manufacturing issues (including documentation) identified in:

### Assembly or handling of the part

### Testing of a part

### Packaging of a part

### Obsolescence issues with parts.

### Supplier concerns with parts and assemblies.

### To update part documentation to enable global sourcing.

### There are other WI/SOP processes that reference the ECR process. See Section titled ”Other Procedures using the ECR Process” for more information.

## Change Review Board Process

### See WI 1063933 for guidance on the Change Review Board Process and process flow.

### CRB may be at the Product Level (e.g., Robotics, Helical, or Software), Functional Level (e.g., Manufacturing Engineering, Service Engineering, and Sustaining Engineering), one CRB encompassing all changes, or another as it makes sense.

#### QMS Changes are generally independent and handle a specific work area. They each have their own timing and details. As needed, an ECR may be transferred or canceled as proposed changes would be handled in a separate QMS CRB by submitting a DCR in the appropriate Document Management System.

### While it is acceptable for each CRB meeting to be independent, consolidation of CRBs is encouraged (as per WI 1063933).

### The Change Owner (or designee) shall populate the Design Change Impact Analysis section of FRM 033017 for all changes to review (whether high or low), to the extent possible prior to CRB review or complete if routed without CRB review. It is encouraged for the Change Originator to support the Change “Assigned To” person in population of FRM 033017.

##### The documented plan shall provide at least the following: Design Verification and Validation, Design Review, Design Transfer, Product Safety and Risk Management, Product Regulatory Compliance, & Change Management.

#### Requests shall include a complete description on the ECR Cover Page, redlines of existing documentation (optional), and a populated FRM 033017.

### Sustaining Engineering

#### This CRB is used to review any issue, concern, or improvement opportunity that has been identified with a PRODUCTION RELEASED product or part. Refer to the following examples.

##### Part obsolescence (OBL) issues. The current supplier change request (SCR) process requires all SCRs to be submitted by Global Sourcing as Sustaining ECRs. If you are aware of an OBL issue, contact Global Sourcing to have them enter the ECR on your behalf.

##### Any manufacturer specification change.

##### Manufacturing issues that impact (changes) the BOM, manufacturing processes, or part yields. Note: BOM maintenance and ASL updates are resolved with the Manufacturing Engineering CRB.

##### Service issues that are related to parts quality, reliability, serviceability, diagnostics, machine interruptions, or other design-related issues.

##### Requests to release a special part/BOM that is currently purchased and defined only within an SCD and not separately released in our system.

##### Any supplier concerns or observations. Typically, the ECR submitter should provide the SCR form. Refer to related procedures in Section 5.6.

##### Regulatory or compliance issues related to a part or product. For example, a customer sale into a new country requires regulatory approvals.

##### Any new action to be considered as an R&D improvement initiative. For example, a collection of associated tasks, such as addressing the "Top 6 OBF".

### Manufacturing Engineering

#### This CRB is used by Manufacturing Engineering to update their processes and perform BOM maintenance.

##### Updates to the Approved Vendor List (AVL) - including corrections to manufacturer's part number or making a second source the primary source, etc. Note: See SOP 016487 for AVL management.

##### BOM Maintenance - Corrections to quantities, usage, and structure as needed by the business to make sure the production line runs efficiently.

##### Updates/Corrections to manufacturing work instructions.

### Service Development Engineering

#### This CRB is used by Service to process requests by field service personnel to create or update service work instructions or FRU BOMs. It may also include other activities internal to service.

##### Request by field personnel to modify/improve a work instruction.

##### Request to modify/correct FRU BOMs based on field data or other observed issues.

##### Split/combine FRU kits to optimize for the needs of the field.

## Process

### See WI 1063933 for a more detailed process flow diagram.

### ECR Triage

Figure 1: ECR Triage Process Flow

#### The triage activity consists of a regular meeting to review newly submitted ECRs and requests for prior ECRs to be re-reviewed (e.g., currently assigned, being investigated, or previously rejected). The ECR will be reviewed against the following items to determine the appropriate response:

##### Incomplete Request: Is there enough information to understand the need, urgency, and to take appropriate action? If not, the ECR will be returned to the originator (Pending state) to be revised and resubmitted.

##### Urgent Work: Is there an issue that requires immediate action? Typical items would be impending part shortages due to obsolescence, regulatory needs, or critical customer issues. This request would be assigned to the person doing the work immediately with a due date assigned as appropriate by the needs outlined in the ECR.

#### Service Escalation: The ECR process is not part of the Service Escalation process. Customer down issues should never be resolved with an ECR. Once the immediate customer down issues are addressed, it may be appropriate to submit an ECR with urgency to prevent future customer down issues, depending on the nature of the problem.

##### Investigation Need: Some ECRs may be observations or may not have a well understood cause or solution. The ECR may be assigned to investigation to better understand the issue and more fully understand the scope, priority, and appropriate owner.

###### The 'lead investigator' is assigned the ECR in Agile for the duration of the investigation state.

###### Once the investigation is complete, the investigator will approve the investigation state to mark it complete, and the ECR will then be reviewed for action and either assigned, transferred, or rejected.

##### Transfer: Some ECRs may be more appropriately assigned to a different process or database OUTSIDE OF THE AGILE ECR WORKFLOW.

###### An ECR Coordinator will enter the pertinent information from the ECR into the other database (including ECR number) and notify the owner/coordinator of that database.

###### The destination database and unique identifier will be entered in the ECR Migrated to Other Database field for future reference. For example, an ECR may be more appropriate as a software anomaly in the anomaly handling database.

###### The ECR is moved to the Transferred state.

###### If the ECR is a better fit for a different CRB, an ECR Coordinator will move the ECR back to the Submitted state and change the ECR CRB field to the CRB suggested by the triage team. The ECR will then be processed by the new CRB. In this case, the Transferred state in Agile and Migrated to Other Database field are not used.

##### Identify Future work: Some ECRs are entered for issues that do not require immediate attention. In cases where the work is scheduled in the future, the ECR is assigned for the system owner to decide when and how it will be implemented. Often these ECRs are related to an upcoming obsolescence issue where the initial investigation indicates that there is time to implement the change without impact to the business. Ideally, multiple related changes can be collected and packaged together in a single release.

### Score and Assign ECR Work

Figure 2: Score and Assign Work Process Flow

### The triage team continues reviewing an ECR if it has passed the prior checks.

#### If the ECR is prioritized high enough to be considered, an ECR Coordinator will assign the ECR to the relevant functional manager/team for future work.

#### If the ECR isn’t high enough or priority is an invalid request, it will be rejected. An ECR Coordinator will move the ECR to the rejected state.

### Assign the Work

#### The ECR triage team has two options if they accept the requested change.

##### Assign the work for implementation in Agile without a specific Assignee. This allows the ECR to exist in the system to be viewed as approved for the affected items, but not driving the next change. This also allows an Agile user to see multiple change requests associated with a given part. These changes are effectively on hold until an event (calendar time, higher priority change, collection of multiple changes, etc.) drives an update to the affected item(s).

##### If possible, (considering priorities and resources) assign the work to a function appropriate Assignee. (This is the preferred path)

### ECR Approval

ECR approval is defined in WI 1063933.

### ECR Implementation

#### ECRs are typically assigned to the individual doing the work and marked as Implemented when the change order is complete via a rule on the Relationships tab. However, in certain cases, the ECR may not result in any change being required. This can occur if the requested change does not impact any aspect of a part that is specified in Agile. An example of this is when an off the shelf component is changing revision but not its part number. Another example is a custom component has a change (such as a color) that is not specified in an SCD.

#### In these cases, the ECR assignee will enter rationale in the ECR comments and ask an ECR Coordinator to manually move the ECR from the Assigned state to the Approved No Change Required state.

#### If there are any other ECRs that list the part as an affected item, it is expected that the other ECR(s) will be addressed with the change. If that does not happen, the individual assigned to the ECR must return to the triage team for their approval to delay the other changes.

### ECR Monitoring: ECRs that are accepted and are in varying stages of completion are monitored in the following method:

#### Urgent work: Urgent work is separately identified and closely monitored on a routine basis. The individuals assigned to the work report directly on the state and timing of the work. This monitoring effort is cross-functional to ensure that the data driving the urgency is current and the delivery dates are in line with the business need.

#### Routine work: Routine work is monitored less often than urgent work and is prioritized by the Engineering team with input from Management.

#### Future work: Identified future work is reviewed at least quarterly by a cross-functional team to update the need and timing of the work.

##### It is expected that some of the future work items would be collected in the course of other project work or other active routine work.

##### Future work will be updated as routine or urgent as needed from the cross-functional review.

#### IB monitoring: Some ECRs affect the Installed base (IB) in a way that a more heavily involved cross-functional team is required to address the issue. In this case, the assignee is typically a project manager, and they prioritize, manage, and close out the ECRs. It is also possible that the scope of an ECR is so large that a formal project is created to address the need, in which case the effort is managed (under higher level PDP/Change Control mechanisms) as a project and is no longer monitored under the ECR process.

### The following types of changes do not require an Initial Change Assessment (033017) form to be completed and shall be processed as if they were Low Impact:

#### Service communication (e.g., support bulletin and existing service work instructions)

#### ECRs determined to be “Approved, no change required”

#### Market expansion changes

### The following types of changes do not require ECRs:

#### Product Builder Changes

#### Changes which fall under formal PDP projects

## ECR Creation Instructions – All CRBs

### The ECR creator is the person responsible for creating and submitting the ECR form. The creator is the individual who is in possession of the information and wants to notify the correct CRB to request/make a change.

### Select Create New > Changes > Change Requests. Or navigate to the item, and click Actions > Create New > Changes. This will automatically add the item to the Affected Items tab.

Figure 3: Create ECR

#### Type = ECR

#### Description of Change – A detailed description of what change is being proposed or clearly identifying a problem statement or issue. It is suggested to include the item number and title for searching purposes. Cannot have N/A or none.

##### Describe part change in detail using Accuray PNs, if possible.

##### If the requested ECR describes or is an output of a product defect it shall reference the respective anomaly which also describes the defect. Not all ECRs require an anomaly, for example updating a tolerance may not require an anomaly if a defect is not present. Product defects are a subset of an anomaly, but an anomaly may not always be from a product defect.

##### This is crucial as the anomaly database is one of the databases of record that Accuray uses to monitor product quality and manage released product surveillance per SOP 1044943 Post Market Surveillance.

#### Reason for Change – A general description of why the change should be made. Cannot have N/A or none.

##### Add relevant dependencies and details for change timing such as:

###### Production schedule demand

###### Date in which purchase orders will not be filled

###### Lead Time of affected item

###### Ability and deadline to purchase more of current component to delay change

#### Product Line(s) – Type in the product line, click on the palette   icon, or use Ctrl+L. Double-click on the product line(s) that applies to the affected items. This choice may contribute to the auto-selection of ECR reviewers.

#### Click Save. The Cover Page tab appears.

#### Select the applicable ECR Type from the drop-down menu based on the affected item Subclass(es). The most common ECR Type for Sustaining ECRs is Item.

| ECR Type | Description |
| --- | --- |
| Engineering | Only change requests for items released to Prototype/Development lifecycle phases. Numbered revisions only; no lettered revisions. |
| Item | Only change requests for parts beyond Prototype lifecycle phase (Pre-Production and Production). Product Docs can be included with parts on this type (Released lifecycle phase). [All Parts, Product Manuals (end user), Software, etc.] |
| Product_Doc | Only change requests for Product Docs released beyond Development lifecycle phase (Released). Parts and other documents cannot be included. (Mfg Work Instructions, Service Work Instructions, Service Notifications, Schematics, Specifications, Standards, etc.). |

#### Workflow = ECR Workflow

#### The creator of the ECR is automatically listed as the Originator, but this can be changed to another Agile ECR Creator if needed.

#### Select the affected Product Platform(s).

#### Select the affected Subsystem. If you are not certain, select Unknown.

#### Select an ECR CRB team and click Save. See Section 5.2 to determine the appropriate CRB team. Note: some fields are specific to the Sustaining Engineering CRB workflow. The additional fields are noted in Section 5.4.2.17.

##### As needed, an ECR may be moved to another more appropriate CRB.

##### If you are not sure which CRB to choose, select Sustaining Engineering, contact an ECR Coordinator, or a Design Quality Engineer. This CRB will review the request and assign it to an appropriate CRB.

#### Select the applicable Business Driver category. You may select more than one, from the below listing of including (but not limited to) categories:

| Business Driver | Description of the Change |
| --- | --- |
| Corrective and Preventive Actions (CAPA) | Identified in a CAPA investigation. |
| Cost of Goods Sold (COGS) | An opportunity to improve the Cost of Goods of a product. |
| Customer Satisfaction | An opportunity to improve the customer's experience with a product. |
| Engineering Documentation | A correction to incomplete, missing, or inaccurate information. |
| Environmental, Health, and Safety | A change that impacts the practices and policies aimed at protecting human health, safety, and the environment from various hazards |
| Feature | Supports a new product or new product capability. |
| Manufacturability | An opportunity to improve the time/ease/consistency of the Manufacturing process or materials. |
| N/A | None of the other business drivers apply. |
| Obsolescence (OBL) | Identifies an item that is no longer available for use in Manufacturing and/or Service. |
| Quality | An opportunity to improve yield, consistency, vendor yield, or other performance issue. |
| Regulatory | Supports activities to maintain agency or other Regulatory compliance. |
| Reliability | An opportunity to improve component life [includes Early Life Failures (ELF)]. |
| Serviceability | An opportunity to enhance the speed, accuracy, efficiency, quality, and/or reliability of repair or routine maintenance. |
| Service Parts Quality | An opportunity to improve the cycles of repair and re-repair [includes Out of Box Failures (OBF)]. |
| Supply Chain | supports producing and delivering products or services from raw materials to final buy level good or service. |
| Global Sourcing | Supports vendor-driven issues or needs outside of obsolescence. |

#### Select a response to the question: Does the change require updates to specifications or requirements?

#### Select a response to the question: Test House Audited Part (THAP)?

##### If an affected item is THAP and on the critical components list, consult with a Compliance Engineer for the ECR and change order. As needed, a separate ECR should be created to document the need to update the appropriate compliance documents based on the change.

#### In the Savings Opportunity? (describe) field, describe any savings opportunity associated with the ECR in terms of dollars, parts consumption, personnel time, reliability, or other likely benefit.

#### The Priority Score and Complexity fields in the ECR Triage Info section are required to be filled out before moving the ECR forward.

##### The Priority Score or Urgency Level is based on a subjective perspective and at the discretion of the Change Owner as to how soon this request should be resolved. Options are “Urgent” or “Not Urgent,”

##### The Complexity of “High” or “Low” comes from populating FRM 033017. Pull and populate FRM 033017 and attach to the Attachments section of the ECR. See Section 5.3.9 for exceptions that do not require FRM 033017.

#### The remaining fields in the Supply Chain and other sections are specific to Sustaining Engineering CRB ECRs. These questions assess the impact of parts availability on Manufacturing, Service, vendors, and Global Sourcing.

#### As needed, collaborate with Global Sourcing, Manufacturing, and Service to enter the following information:

##### Select Yes or No to the question: Could this issue cause Customer or Manufacturing downtime?

##### Select Yes or No to the question: Supplier Change Request? If you selected Yes, additional fields are required.

##### Select Yes or No to the question: Last Time Buy Options available?

##### Select Yes or No to the question: Vendor on Hold? In other words, is the vendor unable to fulfill orders?

##### Select Yes or No to the question: Obsolescence (OBL) from Supplier?

##### Enter the Buyer Manager Name: for the affected part(s).

##### Enter the Commodity Manager Name: for the affected part(s).

##### Enter the Part Obsolescence Date: for the affected part(s). If not applicable, enter 01/01/1900.

##### Enter the Last Time Buy (LTB) Date: for the affected part(s). If not applicable, enter 01/01/1900.

##### Enter the Last Time Delivery Date: for the affected part(s). If not applicable, enter 01/01/1900.

##### Enter the Supply Runout Date in Manufacturing for the affected part(s). Runout in this context is when Accuray is expected to consume the last available part. If not applicable, enter 01/01/1900.

##### Enter the Part Lead Time (days) for the affected part(s).

##### Enter the Monthly Usage in Manufacturing: for the affected part(s).

##### Enter the PO Need by Date: for the affected part(s). If not applicable, enter 01/01/1900.

#### If the change/issue also causes a Service Parts Shortage, enter the following information:

##### Enter the Monthly Usage in Service for the affected parts. This is the historical consumption trend, not future forecast estimates.

##### Enter the Forecast of Usage for the affected part(s). This is the future forecast of consumption, not the historical consumption trend.

##### Enter the Supply Runout Date in Service for the affected part(s). Runout in this context is when Accuray is expected to consume the last available part. If not applicable, enter 01/01/1900.

##### Enter text in the Upgrade Path Available? field to describe the possible upgrade paths for the affected part(s).

##### Enter text in the Alternate FRUs Available? field to list alternate FRU options for the affected part(s).

##### Enter the Estimated Date of Last Time Assembly Used in Manufacturing: field to provide a date (or estimated date) of the last time the affected part(s) was used on a forward production manufacturing unit. If not applicable, enter 01/01/1900.

##### Enter the current Installed Base Size: for the affected part(s).

### Click Save. The ECR will show a Pending state in the top right corner. A Pending state allows users to modify the ECR details, CRB team, Affected Items, and Attachments.

### To make further changes, click on the Edit button and click Save when finished. Changes can be made until the ECR is Submitted.

### Click the Affected Items tab and then click Add. At least one Affected Item is required except in rare special cases.

In the typical case, enter the individual part number that is driving the ECR. Adding assemblies where the part is used is desirable but not required unless the part is only described by a Supplier Control Document (SCD). In that case there often isn’t an Accuray part number and the ECR creator must enter all known parts affected by the issue.

### Click the Attachments tab.  Attach populated FRM 033017. See Section 5.3.9 for exceptions that do not require FRM 033017. Attach any other supporting information to ECR. This may include the supplier change request documents, proposed redlines, current part and proposed part specifications, EPFs, etc.

#### Click Add > Browse for files.

#### Select the document > Upload.

#### Enter a File Description (if desired) such as "Supplier Change Request" or "Proposed Redlines" and click on Save.

### Check the Audit Status by selecting Actions > Audit Status. Note this step is optional.

### Any errors will be listed. Click the error to view a description and a link to its location. After correcting any errors, click Save.

### Submit the ECR by clicking Next Status > Submitted.

#### You may add individuals in the Notify box to be notified regarding this change. Other processes may require additional people be added to the ECR as approvers. See Section 5.6.

#### Click Finish. The ECR moves to the Submitted state. The ECR cannot be edited in this state, but you can click the Pending box on the Workflow tab to return the ECR to the Pending state to make changes and resubmit the ECR.

## Agile Workflow – All CRBs

Figure 4: ECR CRB Agile Workflow

### Submitted State

#### The assigned ECR CRB team reviews the content to verify completeness of the ECR and the appropriate CRB team.

#### If the current CRB team is not appropriate for the ECR, the CRB team changes the state to Transferred, and changes the CRB team of the ECR.

#### Depending on the ECR CRB team, a review may be held before the ECR is moved to the CRB state.

#### If the ECR is not accepted by the CRB team, it is moved to the Rejected state.

#### If the ECR is complete but still requires additional information, it is moved to the Investigation state.

### Rejected State

#### The work described in the ECR was reviewed by the CRB team and was not accepted. The ECR cannot move forward in the workflow. As needed, a new ECR may be created to request the change at a later date or under different business conditions.

#### An ECR can be moved to the Rejected state at any point in the ECR workflow before the Implemented or Approved No Change Required states. If the assignee believes that an ECR should be rejected, they must add the rejection rationale as an ECR comment in Agile, reject the CRB approval of the change, and notify an ECR Coordinator for confirmation. The coordinator will then move the Agile state to Rejected.

### Transferred State

#### The transferred state is used to document the transfer of the change request outside of the Agile system.

#### When the assigned ECR CRB team recommends that the ECR should be transferred, an ECR Coordinator enters the pertinent data into the other database and notifies the appropriate process owner that this request is being transferred.

##### After the ECR is entered into another system and the appropriate owner is notified, an ECR Coordinator notes the database and unique identifier within the database for the ECR and then changes the Agile state to Transferred.

#### An ECR can be moved to the Transferred state at any point in the ECR workflow before the Implemented or Approved No Change Required states. If the assignee believes that an ECR should be transferred, they must add the transfer rationale as an ECR comment in Agile, reject the CRB approval of the change, and notify an ECR Coordinator for confirmation. The coordinator will then move the Agile state to Transferred.

### Investigation State

#### An ECR Coordinator moves the ECR to the Investigation state and adds the required acknowledgers and ECR assignee as the lead investigator.

#### The CRB team assigns a resource and a resource manager as an acknowledger. Assigned resources actively work to complete the investigation. An ECR stays in the Investigation state until the investigation is complete, and the results are reviewed with the CRB team.

#### ECRs that are driven by non-supplier external requests must include a Global Sourcing representative in the Investigation step approval as described in this work instruction.

### CCB State

#### An ECR Coordinator moves the ECR to the CRB state and adds the required approvers.

#### CCB approvers consist of one or more members of the ECR CRB team responsible for the disposition decision of the ECR.

#### The CCB team may also return the ECR to the Pending state if the ECR is missing information or contains incorrect information. The Pending state allows the originator to update and resubmit the ECR with the updates requested by the CRB team.

#### In the CCB State the agenda is pulled for the CRB meeting. If an ECR is not in CCB state in Agile it will not be included in the agenda.

### Assigned State

#### An ECR automatically moves to the Assigned state after all the CRB approvals in Agile are complete. A change order can be created from an ECR using the Save As function. The ECR stays in the Assigned state until the change is released.

#### An ECR may be moved from Assigned to Implemented upon completion of a change order for the affected items. A relationship rule may be used to tie the ECR to the associated change order and automatically move the ECR to Implemented upon change order release.

### Implemented State

#### The ECR is Implemented (work complete) after a change order is released.

#### If the ECR is not automatically moved to the Implemented state after the associated change order is released, an ECR Coordinator can move the ECR to the Implemented state and reference the change order(s) where the work was completed.

### Approved No Change Required

#### If the determination is made that an ECR in the Assigned state requires no change order but the change itself is approved, the "no action" decision should be documented by clicking "Add Reviewers" to the release workflow, documenting the no action rationale, and adding the decision-makers to the ECR Workflow CRB. When all acknowledgements have been submitted, the ECR may be moved to the Approved No Change Required state by an ECR Coordinator.

#### ECR Coordinator will update the ECR information with the rationale provided by the assignee and manually move the ECR to the Approved No Change Required state.

## Other Procedures using the ECR Process

- SOP 1060697

- SOP 016827

- WI 1044497

# Records

All ECR records are maintained in Agile. Refer to SOP 016824 for more information.

# Appendix

N/A